# Part 1

## Question 1

What do you think this link light means?

Answer:

The different-colored link light indicates that the port is currently not forwarding traffic. STP has placed the port into a blocking state to prevent a switching loop while keeping the redundant connection available if needed.

## Question 2

What path will frames take from PC1 to PC2?

Answer:

Frames will travel through the active forwarding path:

PC1 → S1 → S2 → PC2

This path is selected by STP as the best route to reach the destination.

## Question 3

Why do the frames not travel through S3?

Answer:

S3 is not used because one of its trunk ports has been blocked by STP. This prevents traffic from using multiple paths simultaneously, which could create switching loops.

## Question 4

Why has spanning tree placed a port in blocking state?

Answer:

STP blocks one redundant link to eliminate Layer 2 loops. This allows the network to maintain backup links without causing broadcast storms or duplicate frame transmissions.

# Part 2

## Question 1

What do you see happen to the status of the G0/2 port during this process?

Answer:

The G0/2 interface changes through several STP states before it begins forwarding traffic. It transitions from a blocking state, passes through the listening and learning stages, and finally reaches the forwarding state after the topology has converged.

## Question 2

Are any ports showing an orange link light that indicates that the port is in a spanning-tree state other than forwarding? Why or why not?

Answer:

No. After STP finishes recalculating the topology, all required links move into the forwarding state because there is no longer a redundant path that needs to be blocked. The remaining active links are necessary to maintain network connectivity.