### art 1: Configure a Home Wireless Router

#### Step 1: Configure DHCP Settings

- Set the Home Wireless Router IP address and DHCP configuration according to the Addressing Table.

- Limit the DHCP server to assign a maximum of 20 IP addresses.

- Configure the DHCP pool to begin assigning addresses from .3 on the LAN network.

- Set the Internet interface to obtain its IP address automatically using DHCP.

Question: Verify the address. What address did it receive?

Answer:
 The router successfully received an IP address automatically from the DHCP server. The assigned address can be verified on the router's Internet Status page.

- Configure the DNS server with the address specified in the Addressing Table.

#### Step 2: Configure the Wireless LAN

- Configure the 2.4 GHz wireless interface.

- Set the SSID according to the Wireless LAN Information Table.

- Select Channel 6.

- Enable SSID broadcast so wireless devices can discover the network.

#### Step 3: Configure Security

- Enable WPA2-Personal security.

- Configure the wireless passphrase using the value provided in the Wireless LAN Information Table.

- Change the default router administrator password to the required value.

#### Step 4: Connect Clients

- Connect the laptop to the wireless network using the PC Wireless application.

- Configure the Tablet PC and Smartphone to join the wireless network through the Config tab.

- Verify that all devices can:

  - Ping each other.

  - Ping the Web Server.

  - Access the Web Server using its URL.

# Part 2: Configure a WLC Controller Network

#### Step 1: Configure VLAN Interfaces

Log in to the WLC using:

- Username: admin

- Password: Cisco123

Create the following interfaces.

WLAN 2

- VLAN ID: 2

- Port: 1

- IP Address: 192.168.2.254

- Subnet Mask: 255.255.255.0

- Default Gateway: RTR-1 G0/0/0.2

- Primary DHCP Server: Gateway Address

WLAN 5

- VLAN ID: 5

- Port: 1

- IP Address: 192.168.5.254

- Subnet Mask: 255.255.255.0

- Default Gateway: RTR-1 G0/0/0.5

- Primary DHCP Server: Gateway Address

#### Step 2: Configure the DHCP Scope

Create and enable an internal DHCP scope with the following settings:

- Scope Name: management

- Start Address: 192.168.100.235

- End Address: 192.168.100.245

- Network: 192.168.100.0

- Subnet Mask: 255.255.255.0

- Default Router: 192.168.100.1

#### Step 3: Configure External Servers

Configure the RADIUS server:

- Server Index: 1

- Server Address: 10.6.0.254

- Shared Secret: RadiusPW

Configure the SNMP server:

- Community Name: WLAN

- Server Address: 10.6.0.254

#### Step 4: Create the WLANs

Wireless VLAN 2

- Profile Name: Wireless VLAN 2

- SSID: SSID-2

- WLAN ID: 2

- Interface: WLAN 2

- Security Mode: WPA2-PSK

- Passphrase: Cisco123

Enable:

- FlexConnect Local Switching

- FlexConnect Local Authentication

Wireless VLAN 5

- Profile Name: Wireless VLAN 5

- SSID: SSID-5

- WLAN ID: 5

- Interface: WLAN 5

- Security Mode: WPA2-Enterprise (802.1X)

Configure the WLAN to use the RADIUS server for authentication.

Enable:

- FlexConnect Local Switching

- FlexConnect Local Authentication

#### Step 5: Connect the Wireless Hosts

- Connect Wireless Host 1 to SSID-2.

- Connect Wireless Host 2 to SSID-5 using the credentials provided in the WLAN Information Table.

#### Step 6: Verify Connectivity

Confirm that the wireless hosts can:

- Successfully ping the Web Server.

- Communicate with each other through ping.

- Open the Web Server using its URL without any connection issues.