Packet Tracer - Basic Router Configuration Review

Addressing Table

| Device | Interface | IP Address / Prefix | Default Gateway |
| --- | --- | --- | --- |
| R2 R2 R2 R2 R2 R2 R2 R2 R2 R2 R2 R2 | G0/0/0 G0/0/0 G0/0/0 | 10.0.4.1 /24 | N/A N/A N/A N/A N/A N/A N/A N/A N/A N/A N/A N/A |
|  |  | 2001:db8:acad:4::1 /64 |  |
|  |  | fe80::2:a |  |
|  | G0/0/1 G0/0/1 G0/0/1 | 10.0.5.1 /24 |  |
|  |  | 2001:db8:acad:5::1 /64 |  |
|  |  | fe80::2:b |  |
|  | S0/1/0 S0/1/0 S0/1/0 | 10.0.3.2 /24 |  |
|  |  | 2001:db8:acad:3::2 /64 |  |
|  |  | fe80::1:c |  |
|  | S0/1/1 S0/1/1 S0/1/1 | 209.165.200.225 /30 |  |
|  |  | 2001:db8:feed:224::1/64 |  |
|  |  | fe80::1:d |  |
| PC1 PC1 | NIC NIC | 10.0.1.10 /24 | 10.0.1.1 |
|  |  | 2001:db8:acad:1::10 /64 | fe80::1:a |
| PC2 PC2 | NIC NIC | 10.0.2.10 /24 | 10.0.2.1 |
|  |  | 2001:db8:acad:2::10 /64 | fe80::1:b |
| PC3 PC-3 | NIC NIC | 10.0.4.10 /24 | 10.0.4.1 |
|  |  | 2001:db8:acad:4::10 /64 | fe80::2:a |
| PC4 PC-4 | NIC NIC | 10.0.5.10 /24 | 10.0.5.1 |
|  |  | 2001:db8:acad:5::10 /64 | fe80::2:b |

Blank Line - no additional information

Objectives

Part 1: Configure Devices and Verify Connectivity

= Assign static IPv4 and IPv6 addresses to the PC interfaces.

= Configure basic router settings.

= Configure the router for SSH.

= Verify network connectivity.

Part 2: Display Router Information

= Retrieve hardware and software information from the router.

= Interpret the startup configuration.

= Interpret the routing table.

= Verify the status of the interfaces.

Background / Scenario

This activity requires you to configure the R2 router using the settings from the Addressing Table and the specifications listed. The R1 router and the devices connected to it have been configured. This is a comprehensive review of previously covered IOS router commands. In Part 1, you will complete basic configurations and interface settings on the router. In Part 2, you will use SSH to connect to the router remotely and utilize the IOS commands to retrieve information from the device to answer questions about the router. For review purposes, this lab provides the commands necessary for specific router configurations.

Instructions

Part 1: Verify Network Connectivity

1. Were the pings successful? (PC3 → PC4 IPv4 และ IPv6)

Answer:

Yes. Both IPv4 and IPv6 ping requests were completed successfully, confirming that the two PCs can communicate.

2. Were the pings successful? (R2 → R1 S0/1/1 IPv4 และ IPv6)

Answer:

Yes. R2 was able to reach R1 through both IPv4 and IPv6 without any packet loss.

3. Were the pings successful? (PC3 → ISP 209.165.200.226)

Answer:

Yes. The ping reached the ISP interface successfully, showing that external network connectivity is available.

4. Were the pings successful? (PC3 → 64.100.1.1)

Answer:

Yes. The destination responded successfully, indicating that routing beyond the ISP is functioning correctly.

5. Was remote access successful?

Answer:

Yes. The SSH connection was established successfully, and login with the configured username and password was accepted.

.

Part 2: Display Router Information

Step 2

What is the name of the IOS image that the router is running?

Answer:

The router is running the Cisco IOS image that is displayed in the show version output (for example, C1900-UNIVERSALK9-M).

How much non-volatile random-access memory (NVRAM) does the router have?

Answer:

The router contains 255 KB of NVRAM.

How much Flash memory does the router have?

Answer:

The router has approximately 250 MB of Flash memory.

What is the boot process for the router on the next reload?

Answer:

The router will boot using the IOS image stored in Flash according to the current configuration register.

Step 3

How are passwords presented in the output?

Answer:

The passwords are displayed in encrypted form rather than plain text.

What is the result of using show running-config | begin vty?

Answer:

The command starts displaying the running configuration beginning with the first line that contains vty, skipping everything before it.

Step 4

What code is used in the routing table to indicate a directly connected network?

Answer:

The routing table uses the letter C to identify directly connected networks.

How many route entries are coded with a C code in the routing table?

Answer:

There are 4 directly connected routes marked with the C code.

Step 5

What command changed the status of the Gigabit Ethernet ports from administratively down to up?

Answer:

The no shutdown command enabled the Gigabit Ethernet interfaces.

What filtering command would you use to display only the interfaces with addresses assigned?

Answer:

Use:

show ip interface brief | include yes

What is the meaning of the [up/up] part of the output?

Answer:

It indicates that the interface is operational. The physical connection is active (up) and the line protocol is also functioning properly (up).